STUDIO CLEANLINESS IS EVERYONES RESPONSIBILITY

MAINTAINING a high standard of cleanliness is a high priority for the Committee and Teachers.

Please clean your workspace thoroughly after use.  Following all cleaning guidelines is essential to maintaining a safe and functional studio.  All members share responsibility for this, so be sure to allow enough time before leaving to complete these tasks properly.

Thank you for doing your part!

WHEEL CLEAN-UP

Safety First – turn off the power before cleaning.

Recycle clay/Slop Bucket – scrape wet used clay (slop) into your bucket first.

Wheel Cleaning – using a bucket of clean water and a large sponge wipe the pan and wheel head.  Clean the exterior body and the foot pedal, chair if necessary.  Sponge will need to be rinsed frequently in clean water whilst doing this task.

Clean the Floor – pull out the wheel.  Sweep any bits of clay up especially if you have been trimming or using the Giffin Grip.  Mop the floor in front and around the wheel using clean water, mop and bucket.

Other Spaces used – make sure the worktable is cleaned and any other areas used i.e. wedging table, recycling clay area, sink is clean and free from bit dollops of clay.

Bats/Tools – scrap down bats including edges and stud holes after use and return to rightful place.  All studio tools clean and returned.  Heat gun left to cool before returning to case.

Final Step – Place the stool upside down on the wheel head to indicate cleaning is complete.

CANVAS BOARDS

To be cleaned (wiped down with clean sponge) and return.  NO work should be left to dry on these boards.

EXTRUDER

Must be cleaned inside and out after use.  NO clay should be left inside the tube.  Dies cleaned.

SLAB ROLLER

Green protective canvas must remain on roller at all times.  Use the appropriate coloured sheets with clay between the canvas sheets i.e. two white canvas sheets for white clay, two grey canvas sheets for all other clays.  Wipe down sheets if necessary to remove any clay that has become stuck.  Drap to dry otherwise fold up after use.

SINK

Sink should be left clean with no big dollops of clay left.  Wet towels should not be left behind

on the sink to dry.

BATS FOR HANDBUILDING

To be cleaned after use and returned to rightful places.  Bats are NOT to be taken home.

KITCHENETTE

Wash up any crockery, utensils used and put away.  All rubbish in the appropriate wheelie bin outside.  Keep this area clean and hygienic at all times.  NO food should be left out to attract ants, cockroaches etc.  Any food left in the fridge may be disposed of by committee if deemed to be old.